Lab #2

Objectives

- Review lecture notes of discrete-time signal and system

- Do additional exercises on SciLab

Report

- For each function or group of functions that you did, you have to capture the screen as evidence that you did by yourself.

- Then, you add all screen captures in a single word file.

- Finally, you upload your report to BKeL on time. You should down-size the image file to reduce the report file in order to be able to submit in BKeL.

---------------------------------

PART 1. REVIEW OF LECTURE NOTES

  - Elementary Signals

- Unit sample sequence (impulse)

δn=1,∧n=00,∧n≠0

- Unit step signal

un=1,∧n≥00,∧n<0

- Unit ramp signal

urn=n,∧n≥00,∧n<0

  - Signal Properties

- Periodic signal

A signal x(n) is periodic with a period N (N>0) if and only if x(n + N) = x(n), n

- Signal’s energy

Ex=n=-∞∞x(n)2

- Signal‘s average power

Px=limN→∞12N+1n=-NNx(n)2

- Signal Symmetry

Any arbitrary signal can be expressed by the sum of two signal components

xn=xen+xo(n)

Even signal component

xen=12xn+x-n

Odd signal component

xon=12xn-x-n

  - Simple manipulations

- Delay

yn=xn-kk>0

- Advance

yn=xn+kk>0

- Folding

yn=x-n

- Addition

yn=x1n+x2(n)

- Multiplication

yn=x1n.x2(n)

- Amplitude scaling

yn=axn

---------------------------------

PART 2. EXAMPLES & EXERCISES

Exercise 1. Let investigate the following functions on Scilab and briefly report their functionalities and how to use it.

| Functions | Description (will be filled by students) |
| --- | --- |
| plot2d3(…) | Produces a 2D discrete plot where vertical stems represent the amplitude of the signal at each sample point. |
| min(…) | Identifies and returns the minimum value within a vector, matrix, or a set of input arguments. |
| max(…) | Identifies and returns the maximum value within a vector, matrix, or a set of input arguments. |
| subplot(…) | Partitions the current graphics window into a grid of sub-areas, allowing multiple plots to be displayed simultaneously. |
| title(…) | Sets a descriptive text string as the title at the top of the active plot or subplot. |
| xlabel(…) | Assigns a text label to the horizontal axis (X-axis) to describe the independent variable (usually time or samples). |
| ylabel(…) | Assigns a text label to the vertical axis (Y-axis) to describe the dependent variable (usually amplitude). |
| bool2s(…) | Converts boolean logic values (%T or %F) into numerical scalars (1 for True and 0 for False), essential for generating basic signals like unit steps. |
| deff(…) | Allows for the dynamic definition of new functions during runtime using character string expressions for the function's header and body. |

Exercise 2. Try the following scripts on Scilab and report what your understanding after observing the output.

scilab:> n = -5:5;

scilab:> msignal = bool2s (n >= 0);

scilab:> plot2d3(n, msignal)

Exercise 3. Try the following scripts on Scilab and report what your understanding after observing the output.

scilab:> n = -5:5;

scilab:> msignal = bool2s (n == 0);

scilab:> plot2d3(n, msignal)

Exercise 4. Use Scilab to draw the unit ramp signal ur(n) for n=-5:5

Exercise 5. Given a discrete-time signal x(n)={1, 3↑, -2}.

Use Scilab to draw the signal x(n), the odd signal component xo(n), and the even signal component xe(n). Each signal will be drawed by a single plot but they are displayed in a single window. Please use title(), xlabel() and ylabel() to represent the name of each plot.

Exercise 6. Given two discrete-time signals

x1(n)={0↑, 1, 3, -2} and x2(n)={0, 1↑, 2, 3}.

Determine y(n) = x1(n) + x2(n). Then use Scilab to draw x1(n), x2(n) and y(n). Each signal will be drawed by a single plot but they are displayed in a single window. Please use title(), xlabel() and ylabel() to represent the name of each plot.

Exercise 7. Given two discrete-time signals

x1(n)={0↑, 1, 3, -2} and x2(n)={0, 1↑, 2, 3}.

Determine y(n) = x1(n).x2(n). Then use Scilab to draw x1(n), x2(n) and y(n). Each signal will be drawed by a single plot but they are displayed in a single window. Please use title(), xlabel() and ylabel() to represent the name of each plot.

Exercise 8. Given a discrete-time signal x(n)={1, -2, 3↑, 6}.

Determine the following signal and then use Scilab to draw the original signal x(n) and the manipulated signal yi(n). Each pair of plots will be display in a single window. Please use title(), xlabel() and ylabel() to represent the name of each plot.

- y1n=x(-n)

- y2n=x(n+3)

- y3n=2x(-n-2)